Technical reader Performance Bungalow

Cie Karin Vyncke

1) Technical crew

1)	Equipe Technique

General, stage and video : Yoris Van den Houte		+32 (0)476 332685

e-mail: yorisvdh@gmail.com

skype ; yorisvdh

Régie lumière : 	Colin Legras / Joelle Reyns 		+32(0)474 377577

e-mail : fatman@no-log.org

2) General remarks. Stage. Setting.

2)	Description du décor

# Minimum deep onstage : 			              8 m.

# Distance between walls of theatre minimum :	12 m.

Hight minimum below protective grid : 	              4 m.

# Proscenium opening : 			            12 m.

Ideal circumstances (fully equiped theatre, 4 stagehands ) we can built up in one day. working day from 08h . show at 20h. The public space is 4.50m from stage. Public is sitting on tiers.

We bring our stage back curtain 11,90m width/ 3,90m height.

We ask a Grey dance carpet  on the floor that covers 12m wide/ 4m depth. We ask all around black drops minimum 5m height. The portal of the setting is one black top masking border 12m width, from the floor up to the border 3m. stage left/stage right open.

3 aluminiumpipes standing vertically on the floor, attached to the grid or pipes.(brought by the Cie)

Monitors and props on stage dispersed.

1 amplifier + CD-player on stage.

1 table-lamp

We hang legs around ( à l’allemande)

3) Vidéo-projection

We project images from two different sources towards the stage . We use our backdrop curtain as a screen. .We hand 2 beamers on 2.50m distance from the stage and 3m. height.

We ask : 2 beamers  1200ansilumen minimum / 3000ansilumen max.

3 dvd-players

1 video mixing table ( freeze).

2 shutter videos (dmx)

1 TV onstage ; big , old , grey. (connexion bnc avec dvd, son séparé)

4)	Modulation Son

4) Sound

2 speakers MTD115b .

1 subbass on the floor, type MTD 118, amplis and processors

3  CD-players

1 soundmixing-table 6/2/2 with  filter sections  (high/himedium/lomedium/low)

5) Light.

-2,50 m. in front of the proscenium hang up on the same track as the beamers are one track (ours) on witch are fixed  16 Quartz de 500watts .

-above the playing area we hang 8 quartz 500watt

-5 tubes fluos

-2 mini-profiles 150Watt

-small laser beam

these lights we bring with us

we ask :

-1 cyclorama light

-1 profile 1KW ( Robert Juliat 614SX)

-3 Table lamps

-3 microphone stands

-Lightconsole with manuel crossfade

-28 circuits  2Kw.

We bring the colourfilters

6) others.

All around the stage is black dance carpet.

We ask the place to be clean and ‘zero equiped’.

1 dressing room with ironing-board and laundry-iron

shower, washing-machine and drying-machine

7) Technical crew

In a well equiped theatre we can built the stage and the image in one day ;

Therefore we need a crew of 4 persons, responsible for stagebuilding, lights and sound.

Dismantling : 		Crew 			2persons	2 heures

During the show we ask the presence of 1 person .

A precise schedule adapted to the conditions of the theatre will be communicated in accordance with the technical responsable person of the organising part.

Thanks.